INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 4

MPEG VIDEO CODING

ISO/IEC JTC 1/SC 29/WG 4 m74517

October 2025, Geneva

Title: [GSC][JEE6.7] TMCV performances and common test conditions

Authors: Julien Ricard, Gilles Teniou, Stephen Wenger

# Abstract

This document presents the performance of the TMCV and proposes new configurations that can be used as future common test conditions.

# Introduction

This contribution presents the results that can be obtained with TMCV compared to the GPCC and Video anchors.

# Configuration

The configurations are based on 3 videos configurations files :

# First video

bd_0         : 10

qp_0         : 0

codec_0      : hmr

config_0     : ./dependencies/HM-18.0-Rext/cfg/encoder_intra_main_rext.cfg,./dependencies/HM-18.0-Rext/cfg/lossless/lossless.cfg

comp_0       : x,y,z,x_add,y_add,z_add

format_0     : yuv400

packing_0    : planar

# Second video

bd_1         : 10

codec_1      : hmr

config_1     : ./dependencies/HM-18.0-Rext/cfg/encoder_intra_main_rext.cfg

comp_1       : opacity,rot_3,zero,scale_0,scale_1,scale_2,rot_0,rot_1,rot_2

format_1     : yuv400

packing_1    : planar

# Third video

bd_2         : 10

codec_2      : hmr

config_2     : ./dependencies/HM-18.0-Rext/cfg/encoder_lowdelay_main_rext.cfg

comp_2       : f_dc_0,f_dc_1,f_dc_2,f_rest_0,f_rest_15,f_rest_30,f_rest_1,f_rest_16,f_rest_31,f_rest_2,f_rest_17,f_rest_32,f_rest_3,f_rest_18,f_rest_33,f_rest_4,f_rest_19,f_rest_34,f_rest_5,f_rest_20,f_rest_35,f_rest_6,f_rest_21,f_rest_36,f_rest_7,f_rest_22,f_rest_37,f_rest_8,f_rest_23,f_rest_38,f_rest_9,f_rest_24,f_rest_39,f_rest_10,f_rest_25,f_rest_40,f_rest_11,f_rest_26,f_rest_41,f_rest_12,f_rest_27,f_rest_42,f_rest_13,f_rest_28,f_rest_43,f_rest_14,f_rest_29,f_rest_44

format_2     : yuv444

packing_2    : temporal

# Global settings

min_block_size: 16

sh_conversion : 601

trans_position: 1

The next figure shows the generated videos.

Figure 1. Examples of generated videos: Geometry (top left), attributes (right) and color and spherical harmonics (bottom left).

The next parameters have been changed by rate points:

- Video QP: QP_1 and QP_2

- Geometry bit depths for MSB and LSB parts

- Format of the color and spherical harmonics (YUV444 or YUV420).

The next table shows the chosen values.

| Rate | MSB BD | LSB BD | Format_2 | QP_1 | QP_2 |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 | 9 | YUV444 | -8 | 0 |
| 2 | 10 | 6 | YUV444 | -4 | 4 |
| 3 | 9 | 6 | YUV444 | 4 | 12 |
| 4 | 9 | 6 | YUV420 | 8 | 16 |
| 5 | 8 | 6 | YUV420 | 16 | 24 |

The main configuration are built to stay in the same biterate range than G-PCC anchor with:

- MSB and LSB bit depth decreasing according to the wich quality : 10/9, 10/6, 9/6

- Color and spherical harmonics video format in YUV444 for the high quality and YUV420 for the low bitrate

- Fixed QP :

  - QP_1 = {-8,-4,4,8,12}

  - QP_2={0,4,12,16,24}

# Results

## Cinema

## Bartender

## Breakfast

## Man with fruit

## BD-rate

The following table shows the BD-rate compared to video anchor:

| Sequence | BD-rate RGB-PSNR | BD-rate YUV-PSNR | BD-rate YUV-SSIM |
| --- | --- | --- | --- |
| bartender_track | -25.0% | -41.6% | -42.2% |
| breakfast_track | -36.8% | -50.4% | -51.4% |
| cinema_track | -32.2% | -47.3% | -39.5% |
| Average | -31.3% | -46.4% | -44.3% |
| ManWithFruit | No overlap | 45.7% | 1.8% |

The following table shows the BD-rate compared to gpcc anchor:

| Sequence | BD-rate RGB-PSNR | BD-rate YUV-PSNR | BD-rate YUV-SSIM |
| --- | --- | --- | --- |
| bartender_track | -15.4% | -14.2% | 2.9% |
| breakfast_track | -31.6% | -25.5% | -1.1% |
| cinema_track | -21.4% | -17.4% | 7.5% |
| Average | -22.8% | -19.0% | 3.1% |
| ManWithFruit | #VALUE! | #VALUE! | #VALUE! |

Note: The results for the Man with Fruit sequence do not appear consistent and require further study to refine the coding parameters. The image size parameters used by the metric software need to be more studied.

The previous results shows that the proposed configurations outperform the GPCC and Video anchors.

# Conclusion

Based on the previous results, we propose that the previous defined configurations are used for the future common test conditions.